| Mini Project Tube manipulator robot. |  |
| --- | --- |
| Team members Last name; First name; Last name; First name. | Students |
| Prepared by: Name; Signature; Date. | One team member |
| Controlled by: Name; Signature; Date. | Second team member |
| Approved by: Name; Signature; Date. | Supervisor |
|  | Sion, Mai 13, 2026 |
| User Requirements Specification |  |

# Time and Date

| URS ID | Description |
| --- | --- |
| 1.1 | When referring to a TimeStamp, we mean a value like 2026-05-12 08:54:04 into a file, or 2026_05_12_08_54_04 if it needs to be used in a filename. |
| 1.2 | By default, Zurich time shall be used as time zone |

# Simplified Control

| URS ID | Description |
| --- | --- |
| 2.1 | This project shall present for simplified machine control a single page accessible via an icon: test-tube in sidebar |
| 2.2 | A description page shall be integrated and commented in the SDS, Software Design Specification. |
| 2.3 | Simplified control with side-bar option fixed. |
| 2.4 | Simplified control with notebook layout. |

# Machine Startup

| URS ID | Description |
| --- | --- |
| 3.1 | The machine shall start with a standard startup sequence: Clear, Reset, Start, which then transitions to the Stopped, Idle, and Execute states, respectively. |
| 3.2 | A Stop button shall allow switching from the Execute or Idle state to the Stopped state. |
| 3.3 | When the machine is in the Execute state, the X, Y, and Z axes shall be synchronized and ready for a Pick or Place sequence initiated by the operator. |

# Pick and Place Sequence

| URS ID | Description |
| --- | --- |
| 4.1 | The operator shall be able to select a nest number between 1 and 13. |
| 4.2 | The operator shall be able to select a speed between 0.01 and 0.3 m/s and an acceleration between 0.1 and 3 m/s². |
| 4.3 | The operator shall be able to initiate either a Pick sequence or a Place sequence. |

# Log events

| URS ID | Description |
| --- | --- |
| 5.1 | Each action on the simplified control page shall be logged in a logTubeEvent.json file |
| 5.2 | The action log shall contain the timestamp and the button name. |
| 5.3 | It's a plus to be able to include the username, saved in your Node-RED's User Settings in the json object. |
| 5.4 | It’s a plus if buttons are built with customisable subflows. |
| 5.5 | Each alarm and warning shall be logged in separate file logAlarmsAndWarning.json. |

# Trace pick and place

| URS ID | Description |
| --- | --- |
| 6.1 | Each robot movement shall be logged in a separate CSV file. |
| 6.2 | The file name shall be: PickTimeStamp.csv or PlaceTimeStamp.csv. see URS ID 1. |
| 6.3 | This file shall contain the X, Y, and Z position records in [mm], as well as the Sound sensor dimension in [mm] and the ambient temperature in [°C]. Two decimal places are used. |
| 6.4 | The values shall be separated by semi-column |

# Backup parameters

| URS ID | Description |
| --- | --- |
| 7.1 | It shall be possible to store an XML file at any time that contains the list of PackTag.Command.Parameter_Lreal, including the list of nest positions for the tubes. |
| 7.2 | The file name shall be: CmdTimeStamp.xml see URS ID 1. |
| 7.3 | The file shall contain the X, Y, and Z position records in [mm], as well as the Sound sensor dimension in [mm] and the ambient temperature in [°C]. Two decimal places are used. |

# Version

| URS ID | Description |
| --- | --- |
| 8.1 | The software shall provide a list of all the versions needed to load the flows.json on a new machine. |

# Archiving

| URS ID | Description |
| --- | --- |
| 9.1 | The URS, FS, DS, IQ, OQ, and PQ documents shall be submitted in hard copy. |
| 9.2 | A final report and your JSON file shall be archived on a Moodle repository for each group. |